Document 2 — Network Troubleshooting & Incident Runbook

Project Testing / Sample Enterprise Documentation

# Purpose

Provide Service Desk and L1/L2 engineers with a repeatable troubleshooting workflow for common enterprise network incidents. The examples are intentionally generic and safe for lab/project testing.

Note: All ticket numbers, hostnames, users, IPs, business names and timestamps below are fictional sample data designed to resemble enterprise ITSM/Service Desk records. They must be replaced with organization-approved values before operational use.

# 1. Standard Troubleshooting Flow

1) Define scope → 2) Check whether one user/site or many are affected → 3) Verify IP configuration → 4) Test gateway/connectivity → 5) Test DNS → 6) Test target port → 7) Trace route when useful → 8) Check VPN/firewall/load balancer/application dependencies → 9) Capture evidence → 10) Escalate with findings.

# 2. Sample Network Incidents and KB-Style Procedures

| KB ID | Issue | Symptoms | Initial Checks | Decision / Resolution Path | Escalation |
| --- | --- | --- | --- | --- | --- |
| NET-KB-001 | No Internet / Network Connectivity | User reports all websites unavailable | Check cable/Wi-Fi; ipconfig /all; ping default gateway; ping approved DNS; test known internal service; test external connectivity if policy allows | If no valid IP → DHCP/network. If gateway fails → LAN/Wi-Fi. If gateway works but DNS fails → DNS. If DNS works but destination port fails → firewall/proxy/service path | L2 Network |
| NET-KB-002 | DNS Name Resolution Failure | Application works by IP but not hostname | ipconfig /all; nslookup app01.corp.example; ping DNS server IP; compare with working client | Validate DNS server, suffix and record. Flush local cache only when appropriate; retest. Escalate if authoritative/forwarder issue | DNS/Network |
| NET-KB-003 | Application Port Unreachable | Hostname resolves but application connection times out | nslookup target; Test-NetConnection target -Port 443; check route; compare from another network | If DNS succeeds but TCP port fails, investigate firewall path, listener/service and routing | Network/Security/App |
| NET-KB-004 | VPN Tunnel Failure | VPN authentication succeeds but internal resources unavailable | Check client network; confirm VPN IP; ping approved internal gateway; test internal DNS; Test-NetConnection internal target -Port 443 | Determine whether failure is client, VPN gateway, route, DNS or firewall policy | Network/IAM |
| NET-KB-005 | High Latency / Packet Loss | Application is slow or sessions disconnect | Ping approved endpoint; tracert target; collect repeated latency samples; compare wired/Wi-Fi and another user/site | Identify where latency/loss begins; avoid assuming a single intermediate hop is the fault unless end-to-end evidence supports it | Network Operations |
| NET-KB-006 | DHCP Address Failure | Client has APIPA or invalid network configuration | ipconfig /all; verify adapter; ipconfig /release; ipconfig /renew; check VLAN/DHCP scope if authorized | If renewal fails for multiple clients, investigate DHCP scope/service/network path | Network/DHCP |

# 3. Command Reference

| Command | Purpose |
| --- | --- |
| ipconfig /all | View IP address, subnet, gateway, DNS and adapter configuration |
| ipconfig /renew | Request a new DHCP configuration where applicable |
| ping <host> | Test IP/ICMP reachability; ICMP may be blocked by policy/firewall |
| nslookup <hostname> | Test DNS name resolution using configured DNS |
| tracert <host> | Trace the path toward a destination and help identify where routing may stop |
| Test-NetConnection <host> -Port 443 | Test TCP connectivity to a specific port |
| Test-NetConnection <host> -TraceRoute | Run route diagnostics from PowerShell |

# 4. Sample Test Evidence

| Test ID | Scenario | Sample Result | Conclusion |
| --- | --- | --- | --- |
| NET-TC-001 | DNS failure | nslookup app01.corp.example → server timed out | DNS server/path requires investigation |
| NET-TC-002 | Port 443 test | TcpTestSucceeded=False; DNS resolves | Investigate firewall/listener/routing |
| NET-TC-003 | Gateway reachability | Ping gateway successful; Internet unavailable | Continue with DNS/proxy/upstream tests |
| NET-TC-004 | Route issue | tracert stops after branch router | Provide hop/path evidence to Network Operations |
| NET-TC-005 | VPN | VPN connected; internal DNS unavailable | Investigate VPN DNS/route configuration |

# 5. Escalation Template

Incident: INC00xxxxx
User/Site: <sample user/site>
Start time: <timestamp>
Business impact: <impact>
Source IP: <IP>
Destination: <hostname/IP>
DNS result: <result>
Gateway test: <result>
Port test: <result>
Trace result: <summary>
Actions already performed: <steps>
Evidence attached: <screenshots/logs>
Requested team: <Network/DNS/Security/Application>

# 6. Reference Basis

Microsoft documents ipconfig-based IP/DNS validation, ping tests to DNS servers, nslookup for DNS queries, TRACERT for path diagnostics, and Test-NetConnection for connectivity and TCP-port diagnostics. Commands should be used within your organization's security and change-control rules.